[image: image26.jpg]R R E R EIRAE]

SHENZHEN OGEMRAY TECHNOLOGY CO.,LTD





[image: image27.png]RINHAERFEF—BRIRTUBAK=. 0=
Tel:+86 755 33985009 Fax:+86 755 33687178
4/F, 5Building , Dongwu Industrial Park,Donghuan
1st Road, Longhua District , Shenzhen, China

http://www.ogemray.com





[image: image28.jpg]R R E R EIRAE]

SHENZHEN OGEMRAY TECHNOLOGY CO.,LTD




[image: image26.jpg]



All rights reserved by Ogemray

The content can’t be modified without permission in this document
***Historical Release***

	Version
	Modified Content
	Writer
	Auditing
	Date
	Department

	
	
	
	
	
	

	V1.0
	the first version  
	Jasmine
	—
	2025-04-28
	Product dept.


1. Product Instruction
1.1 Product Overview
SW31 is a 2-way combination for 1 gang 16A smart relay switch, which provides energy management and better load capacity for powerful home appliance

The device allows you to power on or off your light/small home appliance anytime and anywhere without the need of any additional hub or controller. Great performance for monitoring power consumption to making your home more secure and convenient.

1.2 Product Function
It is installed in a wall mount to control high power electronic appliance at home.
Works with Alexa and Google assistant, control lamp or small appliance with your voice.
16A relay switch control, support high-power household appliances.
Patent WiFi access technology, users configure network one step intelligently by APP. 

Control lamps and appliance from anywhere by APP with mobile network access

Energy saving: Reduce energy use, eliminate wasteful standby power

Set schedule: Turn on/off your devices based on specific time you set

Real time power monitoring
Smart scenes and automation support.
Multiple user control

Group control

Control history view

Support 2.4GHz WiFi​ 
1.3 Works with Alexa and Google assistant
The smart switch works directly with Amazon Alexa and the Google assistant, you can manage your devices through voice control like turn on/off. Assign a name to each smart switch and communicate with switch by name when making a voice command, Users could purchase these intelligent speakers on the market for convenient usage.

To voice control the smart switches, we need to complete simple configuration in addition. 
Open Alexa APP,search skill “seetime” , complete account authorization, discover new device.

Open Home APP,search service “seetime”, complete linking account.
[image: image1.png]=

Amazon Echo Google Home DuerOS Tmall Genie Xiaomi




2. Specification
2.1 Product Figure

[image: image2.jpg]



2.2 Parameters

	Parameters
	Details

	Data transmission
	WiFi 2.4GHz

	WiFi chip
	ESP8684H2

	Input voltage
	100-240V AC, 50/60Hz

	Rated current
	16A

	Rated power
	3500W

	ON/OFF control
	1 Gang

	Reset button
	Housing button

	Blue indicator
	1PCS

	Dimension
	41*40.5*21mm

	APP support
	Support Android6.0 and 

iOS11.0 or above


3. Operation Introduction
3.1 Wiring Instruction

[image: image3.png]



Description:
	Identity
	I/O
	Description

	N
	Input
	Neutral line, AC neutral line input, output public

	L
	Input
	Live line，AC live line input

	S1
	Input
	Mechanical switch input to turn on/off L1

	L1
	output
	AC output, connect load such as lamp, small appliance.

Rated power: 3500W


3.2 Operation Guide

[image: image4.png]0000

O/ Blue indicator

].—Reset button





Reset the device:

Long press the reset button 5 seconds until the blue indicator flashes quickly.
4. APP Introduction
4.1 APP Download

The APP is compatible with Android 6.0 and iOS11.0 or above system. iOS users could search the APP in App Store. Android users could search the APP in Google Play.

APP name: “Seetime Smart”

[image: image5.png]Andriod
(App Store) (Google Play)

Seetime Smart




4.2 WiFi Configuration

4.2.1 Add device in Easy Mode

WiFi networking configuration for SW31 is convenient and rapid, mobile could be connected to the designated router quickly, the whole process is generally less than 15 seconds. 
Make sure the wiring of WiFi smart switch is completed and it is powered on.(The smart switch only supports 2.4GHz WiFi router)
Configuration process: 

Step 1. Complete wiring, power on the switch, search in App store/Google Play or scan QR code to install APP, connect mobile phone to 2.4G WiFi.

Step 2. Open “Seetime Smart” APP to click “+” to add device.
[image: image6.jpg]No device now

= A

Scenes My




 [image: image7.jpg]16:13

E g,_) e g dl(

A\
T\
VaVa

@ O O 6|0

ail 56 €3

Add Device

Qllidi L bUIlp

Smart lighting controller

Smart Plug

Smart Power Strip

Tempering treasure

Switch

Smart gas switch

WiFi Range Extender with Smart
Plug

Lucky light

Low temperature slow cooker

Calafaccion.da-agua caliente




 [image: image8.jpg]16:13 il 56 E3

< Add Device

-I WiFi Smart Switch

E Smart Panel Switch

Tﬂ Smart reset switch





Step 3.Input current WiFi password,click next step.

[image: image9.jpg]16:14 Q) ol ¥ 63

< Choose home WiFi

Enter your currently connected WiFi password

HUAWEI-0H17M8

II

Next step





Step4. Check device status, and make sure the device has been reset.
When the smart switch is connected to small home appliance, after wiring and device is powered on, please check if the blue indicator flashes fast. If yes, please confirm and click “Next Step” directly. If not, please reset device.
Reset smart switch

Long press the reset button 5 seconds until the status indicator flashes quickly.

Confirm and start connection.

 [image: image10.jpg]16:14 4 al © 83

< Easy Mode

Product pic represent catagory as reference

Complete wiring, power on the device, confirm
blue indicator is flashing.

How to make blue indicator flash.

The blue indicator is flashing.




  [image: image11.jpg]16:14 7 ol ¥ 63

N\

Add Device

7%

distance between Router,phone and Device is not too

far

® Send wifi password to Device

Find the Device

Device initialize




 [image: image12.jpg]— All devices
Unclassified (1) ~ @
-l STswitch_E778 I
Local = U
= 8
Home Scenes My





4.2.2 Add device in AP Mode

AP mode is a supplementary configuration way, if you failed configuration in Easy mode, please try AP mode to add device.

Step1. Select “Add device by AP mode” to add device, and confirm the blue indicator is flashing slowly.
[image: image13.jpg]10:14 ol

)

< Add fail

Device addition in Easy Mode has failed!!!

Please check the possible problems as following
steps and try again

1. Ensure that your device has been restored to
the network configuration mode according to the
initial prompt

2.ensure wifi account and password are correct

3.ensure wifi name not include chinese and
character

4.ensure your router is 2.4Ghz

5.In the easy mode configuration fails, you can
try ap mode ap mode higher success rate

Add device by AP mode




 [image: image14.jpg]10:14 4 ol

)

< AP Mode

(1N

The blue indicator is flashing slowly.





Step2. According to page prompt, go to WLAN setting in your smart phone and select the “STswitch_XXXX” for your WIFI, no need to input password,then back to APP.

[image: image15.jpg]10:16 4 ol

)

4 AP Mode

" 1:51 @ 9 98% mm

¢ Setting Wi-Fi
Wi-Fi O
v STswitch_xxxx

Loading...
Air e ()
cfq5g e ()
juytg e = ()
QA7 10,
RND_5G e = ()

Please let your mobile connect to 'STswitch_xxxx' as soon
as possible and return to the current App

@ Do not switch to other Wi-Fi during connection

PY When returning the current APP, please be patient and wait
for the phone to connect to the device hotspot successfu...

0 Current WiFi:  HUAWEI-OH17M8

Connect to device WiFi




 [image: image16.jpg]15:59 4 al 56 &

< Settings Edit

L 4

WLAN

Connect to WLAN, view available networks, and
manage settings for joining networks and
nearby hotspots. Learn more...

STswitch_E778

Unsecured Network

WLAN ()
=0

MY NETWORKS

0))
®

HUAWEI-0H17M8 a

HUAWEI-0H17M8_5G a8

o))
®

:L;AWEI-OH’I7M8_5G_WI- a = @

RND 8= (0
SeeTime_5G a = (O
Xiaomi_12 a = @
XM 8= (0





Step3. Once you select the “STswitch_XXXX ” and back to app, click [image: image17.bmp]to select WiFi and input password, click “Next Step” to start connection.

[image: image18.jpg]10:27 ol

)

< Choose home WiFi

Enter your home WiFi password

HUAWEI-0H17M8

Next step




 [image: image19.jpg]10:27 ol

)

< Choose home WiFi

Enter your home WiFi password

HUAWEI-0H17M8

HUAWEI-0H17M8 -

TP-LINK_3255

Next step




 [image: image20.jpg]0:27 ol

)

< Choose home WiFi

Enter your home WiFi password

HUAWEI-0H17M8

Next step





Tips: If there is no WiFi around the smart switch, you can select “Direct control” to connect, then the smart plug can be only local controlled by APP, and can’t voice controlled by Alexa/the Google assistant.

Step4. Once devices configured successfully, you will find it in device list.

[image: image21.jpg]‘:28 4 ol ? .

< Try to connect device

22%

Searching Now

Cancel




 [image: image22.jpg]— All devices
Unclassified (1) ~ @
-l STswitch_E778 I
Local = U
= 8
Home Scenes My





4.3 APP Control Interface

Click “device list” to enter device control interface.
[image: image23.jpg]— All devices

Unclassified (1) ~ @

-l STswitch_E778

Local =

= o

Home Scenes My




 [image: image24.jpg]16:31 ol ? @’

< STswitch_E778 cee

real time Power

0 0 1 1 4 1





ON/OFF
Real-time power review
Timing function
Control history view

Power monitoring
Local/remote control
5. Product Update
Smart switch supports wireless silent upgrade（FOTA）.

iOS APP supports App Store update, Android APP supports update remind in APP, users can select to update the smart switch.
The product also support burning firmware through the reserved pin interface of PCBA.

[image: image25.png]



6. Using environment requirements
6.1 Suitable temperature and humidity range:
Suitable work temperature ：-20℃-+40℃
Suitable storage temperature ：-20℃ -+85℃

Suitable working humidity ：20%-85%(No condensing)
Suitable storage humidity ：20%-85%(No condensing)
Note：In order to maintain the service life of the product and ensure the performance of the product, please use and store this product in strictly accordance with the environmental requirements. 

7. Approval and certification
Smart switch has got the following certification 

	Certification
	Approval

	RoHS
	Pass

	FCC
	Ongoing

	CE
	Ongoing


8. Disclaimer 
This specification is a detailed description of series products produced by the company, Please read carefully and fully understand the detailed specification parameters of the product.

All rights reserved by Ogemray, without permission of our company, The company do not allow to totally or partly copy, translate or simplify this specification for any other usage.

We do not provide any guarantees for the product information, parameters, photos or other links in the  specification, users bear the risks.

we declare that this specification or other relevant information without any implied warranties of business assurance or business contract assurance. All information inside is only for operation, product user manual or reference, it can’t be regarded as a basis or complement for any other contract.

We have made reasonable efforts to make this document content accurate and up-to-date.

but our company reserves the right to update or revise the content and its related information at any time without additional notice.




SW31


WiFi Smart Switch (1 Gang)


Product Specification





ITEM： SW31


version：v1.0


date：2025-04-28











[image: image27.png]6 / 14

